Problem Statement 4:

Title: Optimal Dump Packing

Departments Eligible to Register for Problem Statements:

Electronics & Communication, Electrical & Electronics, Instrumentation Engineering, Robotics, Artificial  Intelligence – Machine Learning, Data Science, Data Analytics, Information Technology, Computer Science Streams only.

Description of Problem statement:

Dump packing is a critical but highly manual step in mine planning today. While current tools provide basic  packing functionality, they place a high cognitive and operational burden on users, especially when working with  complex dump shapes. Current methods are based on a two step approach where

1) The dump is first filled with pre-defined “Spot Points”

2) Lanes are pre-planned to each Spot Point.

This approach has a number of limitations:

When small trucks dump to the pre-defined spot points, low spots occur within the dump. It may be impossible  to pre-plan arrival sequences for mixed fleets.

Where the shape of a paddock dump is irregular, there will always be areas that cannot be covered by pre-defined  spot points. The pre-defined spot points must be adequately separated so that dirt piles from prior dumps are  not detected as obstacles (e.g. trucks stop). Due to the above limitations, methods based on the placement of  pre-defined spot points may never match the density of staffed operations.

Alternative methods for filling dumps with spot points:

Staggered Pattern

Radial Fill Pattern

Solution Options:  Propose a better method for packing spot points into polygons to achieve a packing density near to  staffed operations. The solution does not need to support mixed fleets (e.g. may use a standard size for spot  points). Lanes may be pre-planned for the defined spot points.

All proposed solutions must satisfy the following requirements:

The operational limitations of the Trucks must be taken into consideration (turning radius, …). • Multiple Trucks must be able to dump concurrently given sufficient space (space not already covered  by dumps (piles)).

Trucks must not hit each other when working concurrently in a dump.

Trucks must not deadlock when working concurrently in a dump.

• Trucks must dump without any part of the truck extending outside of the defined polygon. • Trucks must not dump material in locations where the material (pile) may isolate sections of the  polygon (block access to sections of the polygon).

• For options that support a mixed fleet, packing density must be maintained for arbitrary sequences of  Trucks with different payloads.

• Truck perception systems may be enhanced to allow Trucks to dump arbitrarily close to prior dumps  (piles).

Truck systems may be trained to recognise piles (semantic segmentation) so that they may act  differently when approaching piles.

Autonomous Dump Pattern with missed Dumps

Staffed Dump Pattern with no missed Dumps

Autonomous Dump Spacing (7.38m)

Staffed Dump Spacing (3.03m)